1. Primary ESP32 Node Connections

- HC-SR04 Ultrasonic Sensor:

  - VCC $\rightarrow$ ESP32 5V (or Vin)

  - GND $\rightarrow$ ESP32 GND

  - Trig $\rightarrow$ ESP32 GPIO 12

  - Echo $\rightarrow$ ESP32 GPIO 13

- GSM Module (SIM800L / Similar):

  - VCC $\rightarrow$ External Power Adapter (GSM modules require up to 2A bursts; do not power directly from the ESP32 pin).

  - GND $\rightarrow$ ESP32 GND (Common Ground is mandatory)

  - TX $\rightarrow$ ESP32 GPIO 16 (RX2)

  - RX $\rightarrow$ ESP32 GPIO 17 (TX2)

- Local Alarms:

  - Active Buzzer (+) $\rightarrow$ ESP32 GPIO 25 (with a current limiting resistor if needed, though active buzzers run well on direct logic).

  - Active Buzzer (-) $\rightarrow$ ESP32 GND

  - Red LED (+) $\rightarrow$ 220$\Omega$ Resistor $\rightarrow$ ESP32 GPIO 26

  - Red LED (-) $\rightarrow$ ESP32 GND

- Camera Trigger Interface:

  - Trigger Out $\rightarrow$ ESP32 GPIO 14 $\rightarrow$ ESP32-CAM GPIO 12

2. ESP32-CAM Node Connections (For Photo Capture)

- VCC $\rightarrow$ 5V Power Supply

- GND $\rightarrow$ Common GND

- Trigger In $\rightarrow$ GPIO 12 (Connected to main ESP32 GPIO 14)

3. FTDI Programmer Connections (Only used for flashing code)

- Note: Disconnect the FTDI after programming.

- VCC (5V/3.3V depending on board) $\rightarrow$ ESP32 VCC

- GND $\rightarrow$ ESP32 GND

- TX $\rightarrow$ ESP32 RX (GPIO 3)

- RX $\rightarrow$ ESP32 TX (GPIO 1)

- (For ESP32-CAM: Bridge GPIO 0 to GND to enter flashing mode).

System Architecture Diagram

+-----------------------------+

|    5V/2A Power Adapter      |

+-------+-------------+-------+

|             |

v             v

+-----------------+      +-----+-------+   +-----+--------+

| HC-SR04 Sensor  |      |   ESP32     |   |  GSM Module  |

|                 |      | Main Node   |   |              |

| Trig -> GPIO 12 |<---->|             |<->| RX -> GPIO 17|

| Echo -> GPIO 13 |      |             |   | TX -> GPIO 16|

+-----------------+      +------+------+   +--------------+

|

| Trigger (GPIO 14 -> GPIO 12)

v

+-----------------+      +------+------+

| Local Alarms    |      |  ESP32-CAM  |

|                 |      |             |

| Buzzer->GPIO 25 |<-----+             |----> Saves to MicroSD

| LED   ->GPIO 26 |      |             |      (Flash Disk via SPI)

+-----------------+      +-------------+

Project Source Code

The project is split into two programs: one for the Master ESP32 Controller and one for the ESP32-CAM capture peripheral.

1. Main ESP32 Controller Code

C++

// Master Controller Code (ESP32-D0WDQ6)

#include <HardwareSerial.h>

// Pin Definitions

#define TRIG_PIN 12

#define ECHO_PIN 13

#define BUZZER_PIN 25

#define LED_PIN 26

#define CAM_TRIGGER_PIN 14

// Thresholds

const int DISTANCE_THRESHOLD = 50; // Intrusion distance in cm

long duration;

int distance;

// Hardware Serial for GSM (UART2)

HardwareSerial gsmSerial(2);

void sendSMS() {

gsmSerial.println("AT+CMGF=1"); // Set SMS format to text

delay(1000);

// Replace with the destination admin phone number

gsmSerial.println("AT+CMGS=\"+1234567890\"");

delay(1000);

gsmSerial.print("SECURITY ALERT: Physical intrusion detected! Camera has been triggered.");

delay(1000);

gsmSerial.write(26); // ASCII code for Ctrl+Z to send the message

delay(5000);

}

void triggerAlarm() {

// Signal ESP32-CAM to take picture

digitalWrite(CAM_TRIGGER_PIN, HIGH);

// Fire Local Alarms (Buzzer & LED)

for(int i = 0; i < 10; i++) {

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(LED_PIN, HIGH);

delay(300);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_PIN, LOW);

delay(300);

}

digitalWrite(CAM_TRIGGER_PIN, LOW);

}

void setup() {

Serial.begin(115200);

gsmSerial.begin(9600, SERIAL_8N1, 16, 17); // RX=16, TX=17

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(LED_PIN, OUTPUT);

pinMode(CAM_TRIGGER_PIN, OUTPUT);

digitalWrite(CAM_TRIGGER_PIN, LOW);

Serial.println("Initializing GSM Module...");

delay(3000); // Wait for GSM to attach to network

gsmSerial.println("AT"); // Handshake

delay(1000);

}

void loop() {

// Clear the trigger pin

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Send a 10-microsecond pulse

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read echo travel time

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance in cm

distance = duration * 0.034 / 2;

if (distance > 0 && distance < DISTANCE_THRESHOLD) {

Serial.print("Intrusion Detected! Distance: ");

Serial.println(distance);

// Execute alert protocols

triggerAlarm();

sendSMS();

delay(10000); // Cooldown to avoid spamming alerts

}

delay(60);

}

2. ESP32-CAM Peripheral Code

This code runs independently on the ESP32-CAM. It waits for its trigger pin to go HIGH, captures an image from the OV2640 sensor, and commits it directly to the onboard Flash Disk (MicroSD card).

C++

// Peripheral Camera Code (ESP32-CAM)

#include "esp_camera.h"

#include "Arduino.h"

#include "FS.h"                // File System

#include "SD_MMC.h"            // MicroSD Card dependency

#define TRIGGER_PIN 12

// OV2640 Camera Pin Configuration (AI-Thinker model)

#define PWDN_GPIO_NUM     32

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM      0

#define SIOD_GPIO_NUM     26

#define SIOM_GPIO_NUM     27

#define Y9_GPIO_NUM       35

#define Y8_GPIO_NUM       34

#define Y7_GPIO_NUM       39

#define Y6_GPIO_NUM       36

#define Y5_GPIO_NUM       21

#define Y4_GPIO_NUM       19

#define Y3_GPIO_NUM       18

#define Y2_GPIO_NUM        5

#define VSYNC_GPIO_NUM    25

#define HREF_GPIO_NUM     23

#define PCLK_GPIO_NUM     22

int pictureCount = 0;

void configCamera() {

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_nda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOM_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

config.frame_size = FRAMESIZE_VGA; // 640x480 resolution

config.jpeg_quality = 10;

config.fb_count = 1;

esp_err_t err = esp_camera_init(&config);

if (err != ESP_OK) {

Serial.printf("Camera init failed with error 0x%x", err);

return;

}

}

void captureAndSave() {

camera_fb_t * fb = esp_camera_fb_get();

if (!fb) {

Serial.println("Camera capture failed");

return;

}

// Create unique filename

String path = "/intruder_" + String(pictureCount++) + ".jpg";

fs::FS &fs = SD_MMC;

File file = fs.open(path.c_str(), FILE_WRITE);

if(!file){

Serial.println("Failed to open file in writing mode");

} else {

file.write(fb->buf, fb->len);

Serial.printf("Saved file to path: %s\n", path.c_str());

}

file.close();

esp_camera_fb_return(fb);

}

void setup() {

Serial.begin(115200);

pinMode(TRIGGER_PIN, INPUT_PULLDOWN);

configCamera();

if(!SD_MMC.begin()){

Serial.println("SD Card Mount Failed");

return;

}

}

void loop() {

// Check if main ESP32 sent high alert trigger

if (digitalRead(TRIGGER_PIN) == HIGH) {

captureAndSave();

delay(5000); // Lockout window to ensure it saves cleanly

}

delay(100);

}